| Project evaluation form – EP EU Project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Customer Name: |  | Mathieu Menant | Project Name: | Blondel Group |  |
| Project Location: |  | France | Judging time: | 2026-03-02 |  |
| Judging participants: |  |  | Initiating Review Department: | Digital Intelligence New Warehouse Division |  |
|  | Job to do |  | ['Stacking/Conveyor', 'Narrow Aisle'] |  |  |
|  | Venue Parameters | 1、Warehouse/ workshop area: |  |  |  |
|  |  | 2, aisle width: | Aisle Width:1.8 m General Aisle: 1.8 m Picking Aisle: mm Unloading Aisle: mm |  |  |
|  |  | 3, Shelf size and height (overall height/single-layer height): | 3.0 |  |  |
|  |  | Clearance Height Under Platform / Obstacles: | m |  |  |
|  |  | 4, the number and size of storage locations： |  |  |  |
|  |  | 5, slope | 0.0 |  |  |
|  |  | 6, others： |  |  |  |
|  | Load Parameters Material Flow | Pallet type/size: (with picture) | Euro Pallet Width (Fork Entry): 1200.0 mm |  |  |
|  |  | weight and size | 600.0 kg Load Dimensions (L×W×H): 1200×800×1110-1210 mm |  |  |
|  |  | Operational efficiency requirements: | Peak Efficiency:Inbound/ Outbound: 574/Day. Peak day, we have 24 hours to process Usually Inbound/ Outbound 123-319/day Operational Peak Hours: 08:00–12:00 14:00–18:00 Shifts per Day: 2 Pallets per Hour (peak): 24 |  |  |
|  |  | Maximum Transport Distance: | 100.0 m |  |  |
|  |  | Minimum Transport Distance: | m |  |  |
|  |  | Average Transport Distance: | m |  |  |
|  |  | Operation process / | Operation process / Material Flow: Flow Sequence: Inbound → Rack Storage → Outbound → Other Inbound: Manual unloading of the lorries and drop off in the staging zone 2. Manual drop off pallets at the inbound buffer storage in front of the rack (0 pallets/hour) Rack Storage: XSC picks the pallet and stacks in the Racks XSC picks from the racks and places it in the outbound buffer area (0 pallets/hour) Outbound: Manually move the pallet from the Outbound buffer to the staging zone (0 pallets/hour) Other: Phase 2: Inbound: • XPL picks the pallet from the staged area and drops the pallet in the buffer area of the assigned rack or manually someone places the pallets from the staging area Outbound: • XQE picks from the racks and stores it in the outbound area front of the racks • XPL picks up and drops it to assigned staging area or manually picks up the pallet and moves to the staging area Phase 3: XNA stacking to all levels. (0 pallets/hour) Flow Sequence: Inbound → Rack Storage → Outbound → Other |  |  |
|  |  | 5 Logistics roadmap: (with pictures) | [Insert Logistics Roadmap / Material Flow Diagram(s) here] |  |  |
|  |  | 6、Other | NA / / 0.0 mm gaps Special Comments / Direct Flows / Notes: |  |  |
|  | Support facilities Parameters | 1、Whether there is a charging pile/battery swap station： | / |  |  |
|  |  | 3、Network coverage: | / Yes |  |  |
|  |  | 4、Positioning accuracy docking requirements, whether it is necessary to dock equipment or assembly lines, etc.: |  |  |  |
|  |  | 5、Whether there are related equipment that needs to be docked (elevators, robotic arms, roller shutter doors, telescopic doors, vertical warehouses, winding machines, etc.), etc.: |  |  |  |
|  |  | 6、Other： | / / |  |  |
|  |  | 1、Explosion-proof level, low/high temperature adaptability, unmanned degree (fully automatic/semi-automatic), remote monitoring requirements, customized appearance and logo, etc： |  |  |  |
|  | Special demand |  |  |  |  |

|  | DATA FLOW scenario description | POC: How to do as small as possible? Inbound: PDA Scan QR, Scan Label, click inbound. DAS selects storage location. Outbound: ? Stand alone? Inbound: PDA->DAS (EP WMS)->USP (Fleet Management)->AGV (Task Completion) Outbound: Dashboard material select->DAS->USP->AGV (Task completion) ->How much time needed for simple integration to trigger the task by customer WMS? Phase 2/3: Inbound: PDA-> Blondel WMS-> DAS (EP WMS)->USP (Fleet Manager)-> AGV (Task Completion) Outbound: Dashboard-> Blondel WMS-> DAS (EP WMS)->USP (Fleet Manager)-> AGV (Task Completion) Customer WMS: Reflex Integrator: Hubone (EP contact) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Connections / Interfaces to Other Equipment: | ['Fire Alarm', 'Automatic Doors'] |  |  |  |  |
|  | Details on Connections / Interfaces: |  |  |  |  |  |
| Specifications and quantities of products to be used： |  | Specifications: |  |  | Specific customization |  |
|  |  | XQE122 / XSC |  | Truck type depending on delivery time and EP technical preference |  |  |
|  |  | XPL201 |  |  |  |  |
|  |  | DASP |  |  |  |  |
|  |  | XNA |  | Payment terms |  |  |
| Business Terms |  | Total project price |  |  | Payment terms |  |
|  |  | Project delivery time |  |  |  |  |
|  |  | Acceptance conditions |  |  |  |  |
| Key technical points： |  |  |  |  |  |  |
| Risk points： |  |  |  |  |  |  |  | Pre-sales implementation representative |
| Project team： |  | Project manager |  |  | Sales representatives |  |
|  |  | Technical representatives |  |  | Sales representatives |  |
| Project plan： |  | Production cycle |  |  |  |  |
|  |  | Implementation cycle |  |  |  |  |
| Final review conclusion： |  | □ approved□ rejected□ returned, re-reviewed □ postponed, and postponed |  |  |  |  |
| Suggestions for improvement： |  |  |  |  |  |  | Software Review： |  |  |
| Signature of the assessee: |  | Product review： |  |  | Production Review： |  |
|  |  | Scheduling review： |  |  | Production Review： |  |
| Review and approval: |  |  |  |  |  |  |