1. Product Scope

| Model | Category | Rated Load | Lift Height | Truck Size (L × W × H) |
| --- | --- | --- | --- | --- |
| XNA121 | Dual-sided narrow-aisle stacking robot | 1.2 t | 8500 mm | 3150 × 1540 × 4465 mm |
| XNA151 | Dual-sided narrow-aisle stacking robot | 1.5 t | 13000 mm | 3400 × 1540 × 2132 mm |

2. Travel Aisle

| Operation Type | Aisle Type | XNA121 Min. Width | XNA151 Min. Width |
| --- | --- | --- | --- |
| Straight Travel / Pick-up / Drop-off | Single aisle — single or both sides | 1740 mm | 1740 mm |
| Turning — aisle change | Single aisle right-angle turn | 4200 mm | 4400 mm |
| Turning — aisle change | Double aisle right-angle turn | 6140 mm | 6340 mm |

3. Turning / Aisle Change

| Model | Truck Width (mm) | Truck Length with Load (mm) | Diagonal Length (mm) | Min. Aisle Switch Width |
| --- | --- | --- | --- | --- |
| XNA121 | 1540 | 3300 | 3700 | ≥ 4200 mm |
| XNA151 | 1540 | 3550 | 3900 | ≥ 4400 mm |

4. Rack Aisle & Dimensions

Rack Installation Requirements

Per EN 15512 / EN 15620 / EN 15629 / EN 15635. XNA operates at high bay heights — rack precision is critical for fork alignment.

| Item | Requirement |
| --- | --- |
| Upright verticality | Individual upright deviation ≤ H/1500, max ±6 mm (e.g. 12 m rack → ≤ 6 mm) |
| Column top parallelism (same aisle) | ≤ ±5 mm |
| Adjacent beam height error | ≤ ±1.5 mm |
| Cumulative beam height error (per row) | ≤ ±5 mm |
| Single beam levelness (end to end) | ≤ 2 mm |
| Overall beam level (same layer, span direction) | ≤ 3 mm |
| Aisle width tolerance | +5 mm / −0 mm — prefer wider, never narrower. Control within W ±5 mm |
| Rack symmetry deviation (both sides of aisle) | ≤ ±3 mm |
| Single rack frame diagonal difference | ≤ 10 mm |
| Straightness — entire rack row (within 30 m) | ≤ ±10 mm along aisle direction |

EU Pallet — 800 × 1200 mm

| Beam Length | Rack Depth | Pallets / Beam | Pallet Spacing |
| --- | --- | --- | --- |
| 2700 mm | 1100 mm | 3 pcs | 75 mm |
| 3600 mm | 1100 mm | 4 pcs | approx. 80 mm |

EU Pallet — 1000 × 1200 mm

| Beam Length | Rack Depth | Pallets / Beam | Pallet Spacing |
| --- | --- | --- | --- |
| 2700 mm | 1100 mm | 2 pcs | 233 mm |
| 3600 mm | 1100 mm | 3 pcs | 150 mm |

Load Placement Requirements

Operating conditions — not recommendations. Loads outside these limits cause task rejection or collision risk.

| Item | Requirement |
| --- | --- |
| Max pallet dimensions (L × W × H) | ≤ 800 × 1200 × 1200 mm or ≤ 1000 × 1200 × 1200 mm |
| Lateral clearance — load to rack face | ≥ 100 mm — recommended ≥ 150 mm |
| Spacing — adjacent racks or rack to wall | ≥ 200 mm — fork tips must not extend more than 50 mm beyond pallet |
| Material clearance between rack levels | ≥ 150 mm (200 mm recommended) |
| Load overhang beyond truck body | Not permitted — special LIDAR configuration required if exceeded |

5. Handling Efficiency

| Parameter | XNA121 | XNA151 |
| --- | --- | --- |
| Forward travel speed (loaded / unloaded) | 2 m/s | 2 m/s |
| Reverse travel speed (loaded / unloaded) | 1 m/s | 1 m/s |
| Mast lifting speed (loaded / unloaded) | 0.28 / 0.28 m/s | 0.28 / 0.28 m/s |
| Mast lowering speed (loaded / unloaded) | 0.38 / 0.33 m/s | 0.38 / 0.33 m/s |
| Pick-up / deposit detection time | 2 s | 2 s |
| Time per aisle turn | 100 s | 100 s |
| Pick-up / deposit cycle time | 106 s | 106 s |

6. Charging Area

| Item | Single Charger | 5-Charger Setup |
| --- | --- | --- |
| Bay width | 1732 mm | 1732 × 5 = 8660 mm |
| End clearance (each side) | 400 mm | 400 mm each side |
| Total width required | 2532 mm | 9460 mm |
| Approach corridor depth | 3181 mm | 3181 mm (shared) |
| Parked truck length | 3300 mm (XNA121) | 3300 mm (XNA121) |
| Rear clearance | 800 mm | 800 mm |
| Total depth required | 7281 mm | 7281 mm |
| Total floor area | ~ 18.4 m² | ~ 68.9 m² |
| Trucks charging simultaneously | 1 | 5 |
| Approach lane | Dedicated | Shared — 1 truck at a time |
| Number of charger units | 1 | 5 |