1. Product Scope

| Model | Rated Load | Lift Height | Truck Size (L × W × H) |
| --- | --- | --- | --- |
| XQE122 | 1.2 t | 5.5 m | 2426 × 1240 × 3053 mm |

2. Mast Options

Confirm lowered height against every door, gate, and overhead obstacle on the travel route before order confirmation.

| Spec | Mast | Lift (mm) | Lowered w/ Nav LiDAR (mm) | Lowered w/o Nav LiDAR (mm) | Free Lift (mm) | Max Height (mm) | Lift Speed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Standard | 3-F | 4500 | 2718 | 2393 | 1368 | 5523 | 0.17 m/s |
| Option | 2-F | 3200 | 2965 | 2640 | — | 4240 | 0.17 m/s |
| Option | 2-F | 3200 | 2823 | 2498 | — | 4173 | 0.17 m/s |
| Option | 3-F | 5500 | 3053 | 2728 | 1700 | 6825 | 0.17 m/s |

3. Travel Path

| Load Carrier | Operation | Aisle Type |  | Min. Width | Remarks |  |
| --- | --- | --- | --- | --- | --- | --- |
| Euro Pallet / Cage / Bin | Straight Travel | Single direction |  | 1600 mm | Speed ≤ 0.3 m/s |  |
| Euro Pallet / Cage / Bin | Straight Travel | Two-way traffic |  | 3360 mm | Controlled — one truck at a time |  |
| Euro Pallet / Cage / Bin | Turn | Single aisle (90° turn) |  | 2450 mm |  |  |
| Cage / Bin 1200 × 1200 mm | Pick-up / Drop-off | Single aisle — single-side |  | 3000 mm |  |  |
| Cage / Bin 1200 × 1200 mm | Pick-up / Drop-off | Single aisle — dual-side (two main aisles) |  | 3000 mm | One aisle between each rack row |  |
| Cage / Bin 1200 × 1200 mm | Pick-up / Drop-off | Single aisle — dual-side (one main aisle) |  | 3520 mm | Both sides from one aisle |  |

4. Turning / Aisle Change

| Area | Minimum | Recommended |
| --- | --- | --- |
| Aisle change clear space | 2900 mm | 3150 mm |

5. Dense Stacking Channel

| Item | Value | Note |
| --- | --- | --- |
| Truck width | 1240 mm |  |
| Clearance — all sides (left, right, front, back) | 200 mm | From load face on each side |

Confirm fork length and pallet depth per project before quoting channel dimensions.

6. Rack Aisle & Dimensions

Rack Aisle Required

| Load Carrier | Operation | Aisle Type | Min. Width | Remarks |
| --- | --- | --- | --- | --- |
| Euro Pallet / Load ≤ Euro Pallet | Pick-up / Drop-off | Single aisle — single-side | 2900 mm |  |
| Euro Pallet / Load ≤ Euro Pallet | Pick-up / Drop-off | Single aisle — dual-side (two main aisles) | 2900 mm | One aisle between each rack row |
| Euro Pallet / Load ≤ Euro Pallet | Pick-up / Drop-off | Single aisle — dual-side (one main aisle) | 3520 mm | Both sides from one aisle |

If pallet overhangs rack face, measure 200 mm clearance from pallet edge — not rack face.

Minimum distance between two adjacent rack rows (face to face): 200 mm.

EU Pallet — 800 × 1200 mm

| Beam Length | Rack Depth | Pallets / Beam | Pallet Spacing |
| --- | --- | --- | --- |
| 1900 mm | 1100 mm | 2 pcs | 75 mm min — 100 mm recommended |
| 2700 mm | 1100 mm | 3 pcs | 75 mm min — 100 mm recommended |
| 3600 mm | 1100 mm | 4 pcs | approx. 80 mm |

Industrial Pallet — 1000 × 1200 mm

| Beam Length | Rack Depth | Pallets / Beam | Pallet Spacing |
| --- | --- | --- | --- |
| 2700 mm | 1100 mm | 2 pcs | 75–100 mm |

Rack depth minimum 1100 mm across all configurations.

Load Placement Requirements

Operating conditions — not recommendations. Loads outside these limits cause task rejection or collision risk.

| Item | Requirement |
| --- | --- |
| Load overhang | Not permitted on any side |
| Load edge to pallet edge (all sides) | ≥ 50 mm |
| Centre of gravity height (from fork surface) | ≤ 900 mm |
| Stacking alignment deviation | ≤ 10 mm |
| Side clearance inside rack (clear space) | ≥ 75 mm — recommended ≥ 100 mm |
| Vertical clearance between rack levels | ≥ 150 mm |

7. Charging Area

| Item | Single Charger | 5-Charger Setup |
| --- | --- | --- |
| Bay width | 1732 mm | 1732 × 5 = 8660 mm |
| End clearance (each side) | 400 mm | 400 mm each side |
| Total width required | 2532 mm | 9460 mm |
| Approach corridor depth | 3181 mm | 3181 mm (shared) |
| Parked truck length | 2426 mm | 2426 mm |
| Rear clearance | 800 mm | 800 mm |
| Total depth required | 6407 mm | 6407 mm |
| Total floor area | ~ 16.2 m² | ~ 60.6 m² |
| Trucks charging simultaneously | 1 | 5 |
| Approach lane | Dedicated | Shared — 1 truck at a time |
| Number of charger units | 1 | 5 |

Plan charging position into the layout from the start — cannot be added after rack layout is fixed.